PASSAGE PLANS: SSV Robert C. Seamans, Voyage S-290

(All times UTC +13 unless noted)

LAST UPDATE: 2/16/20 (Elliot)

Leg I: Auckland to Bay of Islands

150 nm. February 17-21

Total Charts:

NZ 51, 512, 5114, 5121, 52, 521, 522, 5214, 5215, 53, 54, 522, 5227, 531, 532, 533, 5321, 5322, 5323, 5324, 5327

Auckland Departure: 0830 17 February

(Charts NZ 532, 5321,5322, 5324)

Local variation: 20E

Harbor Protocols:

Approach:  VTS required      Pilot: No

Call "Auckland Harbor Control" (Ch. 12) 2 hours prior to departure.

(ETD, draft. POB, Destination)

Departure Conditions:

- PD Checklist complete, including steering and propulsion

- Call to VTS

- Anchors ready (Chief Mate)

- Boat check coverage set (A Watch)

- Lookout system established (A Watch)

- Boat ready and operator briefed, where needed

- Single up, gangway stowed, fenders ready

Notes: B Watch primary sailhandling

NATURAL CONDITIONS ON DEPARTURE AUCKLAND 17 FEB 2020

AUCK TIDES 17 FEBRUARY 2020:

| High tide | 3.2 m | 2:44 AM NZDT |
| --- | --- | --- |
| Low tide | 0.8 m | 8:45 AM NZDT |
| High tide | 3.1 m | 3:05 PM NZDT |
| Low tide | 0.7 m | 9:14 PM NZDT |

Tidal Currents:

To 2+ knots. Ebb starts 45 min. past HW

Flood starts 4:45 before HW (1020)

Forecast: Waitemata Harbour and the Hauraki Gulf

(Issued 0500 Sunday 16 Feb 2020, Valid to 2359 Sunday 16 Feb 2020)

Warning: Strong wind advisory in force for the area from Bream Head to Cape Colville.

## Situation

A deep ex-tropical low approaches from the northwest today, passing across the deep south tonight. An associated front weakens over the country on Monday, then a weak ridge builds over the country during Tuesday and Wednesday.

## Forecast

Sunday: Northeast 15 knots. Sea slight. Cloudy periods, chance shower or two.

## Outlook until midnight Wednesday

Monday: Northeast 10 knots. Partly cloudy.

Tuesday: Variable 5 knots, but daytime sea breeze to 10 knots. Mainly fine.

Wednesday: Variable 5 knots, but daytime sea breeze to 10 knots. Mainly fine.

0830-0900: Depart berth for Anchorage, Auckland Harbor

Route to Motuihe Anchorage (see plot for exact values)

- c/c 096T on leaving, .15 nm clear of wharves

- c/c 055T abeam R “20” 	(2 nm)

- c/c 065T abeam R “16”	(1.2 nm)

- c/c  082T abeam “Y”	(0.6 nm)

- Proceed to anchorage west of Motuihe Is., as shown. (4.3 nm)

18-21 February: Transit Hauraki Gulf, passage plan per conditions. Route to follow Cawthron sampling locations as noted on plotter.

Bay of Islands: (Charts NZ 512, 5124, 5125)

21 February 1100: Arrive, Russell

23 February- Depart for sea, 1100

Bay of Islands Natural Conditions:

- Winds: Prevailing NE, may be strong with passage of depressions.

- Ocean Currents: Prevalent SE set to 1.5 knots

- Tides: Semidiurnal, 2m range at springs

- Tidal currents negligible except 2+ knots in tickles and river entrances.

- Pilotage: Compulsory > 500GT, Notice of arrival 72 hours if >100GT

PASSAGE NOTES

Anchorages near Auckland (as marked)

- Waitema Bay (Auckland Hr.) 0ff Stanley Point. (S270)

- Barge anchorage W. of Rangitoto?

- S of Whangaparoa Penn. (Manley Beach)

- NE side of Motuihe Island (7NM). *Stay outside of 10m curve to avoid clay

- SW side of Motuihe

- E of Musick Point at Tamaki Strait Entrance

- Man-of-War Bay- N end Waiheke Chan. -20 NM  (*excellent –SNZ)

- Mahurangi Hr.  Whangaparoa Bay -15 NM

- Kawau Bay (*good –SNZ, S270)

- Bon Accord Harbor, W side of Kawau Is. -30 NM

- Coramandel Hr. -40 nm

- Tamaki Strait under lee of Waiheke Is. In NE gales, close inshore at Hitapa Bay

Anchorages in Hauraki Gulf and en route to BOI (as marked)

Great Barrier Island

- Port Fitzroy, (W coast GBI) is a very sheltered collection of bays and coves with several anchorage options.  (*can be squally inside- ASD) Good Hiking options ashore. (SNZ advises southern anchorage over Kairara Bay to avoid snags from old logging ops.

- Whangaparapara Harbor- S coast GBI. Open to S quad, can be squally (ASD)

- East coast of GBI: Harotaonga Bay.  Is beautiful and has a long sandy beach,

Sailing east from Auckland or the Gulf to the east side of Great Barrier, Watch for breaking seas in Colville Channel. (Worst in Strong SW’lies.)

- Whangerei

- Whangaruru Harbor: Chart 5111 Anchor inside past cable area, just short of beacons. Good holding. (SNZ)

- Whangamumu (Old Whaling Sta, just S of Cape Brett- good for S270)

- Whangaroa (Landlocked Fjord, north of BOI) Anchor first bay on your right for beauty, further in for best shelter from Easterlies. Some tidal current in entrance, but probably not an issue for RCS most of the time.

Bay of Islands Anchorages:

- Te Rawhiti Inlet

- Te Puna Inlet (Less visited, maybe a good hidey hole)

- Parekura Bay (shelter from all quadrants)

- Rangihoua Bay

- Russell Anchorage (see plan)

- Onewhero Bay

CONTACTS

Auckland

Craig Roe (agent)

craig@rys.co.nz

+64 0274 944 092 mobile

+64 09 368 9301

Nigel Wright, SNZ Marine Superintendent

narmwright@orcon.net.nz

021517263

Georgina MIcet, SNZ Captain

georgina@maggiedixon.co.nz

Gerard Prendeville (Captain, Spirit of NZ)

prendeville@vodafone.co.nz

021 502 311

Russell/Bay of Islands

Russell Harbormaster (Jim Lyle) 	jiml@nrc.govt.nz

+64 9-402-7516

VHF 16

+64 9 438 4639 or 0800 504 639 (Emergency)

Capt. Jim Cottier (Compass adjuster, Roberton Island)

021 179 3170

Opua Marina

Email: enquiries@opuamarina.co.nz

Phone: +64 9 402 7124

Physical: 28A Baffin Street, Opua 0200

Leg II: Russell to Wellington via Great Barrier Island

700 nm, February 23- March 13. (19 days)

Charts: NZ 46, 463, 4633, 4634, 512, 5124, 5125, 52, 53, 534, 54, 541, 5412, 542, 5423 55, 5571, 56, 5612, 57, 58, 6152, 6153, 6212.

Notes:

Route contingent on weather.

Lower half of North Island E Coast (Castlepoint forecast area) is subject to strengthened westerlies due to land forcing.

Harbors of Refuge: North and East Coasts

Tauranga (Bay of Plenty) 54, 541, 542, 5413

- Large commercial port and city at head of Bay of Plenty.

- Strong tidal streams in approach, timing is necessary. Pilotage  >100T

Hicks Bay, Just behind E Cape (S270)

Anchorages either side of Mahia Peninsula between Hawke Bay and GIsborne

Gisborne (Poverty Bay) 5571

- Commercial Port with no anchorage, limited in strong E and S’ly conditions. Pilotage >500T

Napier (Hawke Bay) 561, 5612

- “Breakwater Harbor”- Commercial port, pilotage compulsory >40m LOA. Limited by surge. Spirit does turnarounds here. RCS Visited in 2018 (S283)

- Anchorage possible N of Napier in Hawke Bay

(S270 anchored south of Flat Rock, north of Napier Roads)

- Contacts:

  - Trevor Morrison, Senior Pilot: TrevorM@napierport.co.nz

  - Helen Jackson, Port Admin:   HelenJ@napierport.co.nz

Castlepoint Bay- 57 (plan)

- Bight with shelter to SW with good holding (SNZ)

Wellington and Cook Strait

NZ 46, 463, 4633, 4634. Admiralty Pilot, Central Region Cruising Guide

Planned arrival: TBD. Probably mid-day- 1100 at buoy/1300 at dock

ETA Required 24 and 4 hours in advance, with 1hr. confirmation.

Wellington Contacts:

Harbour Master:  	harbours@gw.govt.nz

+64 4 830 4160 or 0800 496 734

Pilots: 		duty.pilot@centreport.co.nz  or pilot@centreport.co.nz

+64 4 495 3802

Tides (Wellington, 13 March 2020)

Approach

Slack water at Wellington Entrance begins an hour before high tide. Note that current is northerly through Cook Strait while tide is ebbing at Wellington.

NW Gales localized, southerly gales affect whole strait. SE gales strongest near Cape Campbell, (South Island) and north parts of Marlborough Sounds. Entire strait may not be approachable in gale southeasterlies.

Recommend 5nm clearance off of Cape Palliser on approach.

Strong tidal rips, beware of Karori and Sinclair Rips off Cape Terawhiti, SW of entrance. Slack water passage recommended. (Slacks here occur about 1 hour before high and low water at Wellington)

Clear and open to south, possible breakers in harbor entrance during southerly gales. Beware of gusty downdrafts off of high ground around harbor. Watch for floating logs and debris inside. Extensive ferry traffic in approach to two dock locations.

“Beacon Hill” is local name for harbor control. (VHF 16/14) They are a useful source for warnings of sudden wind shifts from North to South.

Wharf: “Queens Wharf”. No agent. Best inside south finger, backed in port side to. OK without fenderboards. (Berth #2) Outer face an option, more wake from ferries.

Special Notes for Wellington Approach:

Wellington is a busy commercial harbor with active harbor control, and a ton of ferry traffic. Ferries have right of way per “narrow channel” rule in COLREGS. Our normal berth is at Queen’s Wharf #2, Unless we are relocated this will mean stern in, port side to. Mates should read the Harbor Bylaws booklet in the passage plan folder. This provides contact information and ETA requirements, as well as the routes that vessels over 18m are advised to follow in their transits. Harbor control will intervene if we stray significantly from these tracks.There is plenty of room to pass in the channel if you are careful.

The approach itself is open, well-marked, and holds few secrets not described in the Admiralty Sailing Directions. Most aids to navigation are fixed beacons, some smaller in size than expected. The main leading marks are unlit by day, and not that conspicuous.

Other Harbors Near Wellington:

Port Underwood (NZ 6212, ASD 4.236)

Large bay on west side of Cook Strait opposite Wellington offers several good anchorage options, settlement and fishing pier at Oyster Bay. Watch for mussel farms.

Queen Charlotte Sound and Tory Channel (615, 6152, 6153)

Numerous anchorages, some are windier than others. See cruising guide and notes on chart for help.

Note very strong tidal currents at Tory Channel Entrance, with reporting requirements on approach. Slack water (turning to fair tide) is at about 5 ½ hours past high tide at Picton, or an hour BEFORE high tide at Wellington.

Ship Cove, (Queen Charlotte Sound) Excellent per SNZ

Grove Arm- less windy, several options

Distances from Wellington

Tory Channel Entrance Cut		40

Ship Cove					60

Grove Arm					56

Port Underwood				45

Leg III- Wellington to Lyttleton

200 nm. March 17-22. (5 days)

Charts: NZ 46, 463, 4633, 4634, 615, 6152, 6153, 62, 6212, 63, 632, 6321, 6324, 64, 6422, 66, 661, 6612,

Harbors en route from Wellington to Lyttleton:

Kaikoura Peninsula, (6212) ASD p. 287

-Several good anchorage alternatives, with landings and shore services. CG station at Kaikoura. About halfway from Wellington to Christchurch.

Abundant birds and marine mammals due to upwelling

Lyttleton & Surroundings (6321)

Approach

-Prevalent NE and SW

-NW gusty and unpredictable

-Difficult in SW gales which blow violently out of the entrance.

-Dryest part of NZ, Jan-March generally hot and settled

ETA: 24 and 4 hours in advance. Pilotage > 500GT and 40m LOA.

(Call: Lyttleton Harbor Radio)

Numerous temporary anchorage possibilities in harbor. (ASD p. 294)

Good anchorage with shelter to S nearby at Port Levy and Pigeon Bay

Wharf: Secure dock, location TBD. Agent is Craig Randall

Craig Randall

Mobile +6421766381

Personal Email craig@nzship.com

Ben Keys ben@nzship.com

Group Email nzsops@nzship.com

Craig Smith was our contact for Independent Provedoring. They can do all provisioning, arrange for fuel, engineering needs, and assist in just about any way. Friendly and excellent. Their office is just outside of the secure wharf area. They have a rental car which is available. There is also an apartment for crew if the boat is late arriving or someone needs to stay after the boat leaves.

craig@indprov.co.nz  web site: www.indprov.co.nz

Phone: +64 3 328 8598

Mobile: +64 21 999 141

Akaroa Harbor (6324) –Good landlocked anchorage, can be subject to violent squalls if windy. Used by cruise ships.

En route from Banks Peninsula to Otago: (NZ 64)

Timaru Harbor: (6422) Large commercial port with artificial harbor about halfway from Banks to Otago.

Omaru Harbor (6433) Small craft harbor, local knowledge required. Anchorage not recommended. Interesting old Victorian town.

Otago Harbor (Dunedin, 661, 6612)

ETA: 72, 24, 12, 4 hours. Otago Harbor Control VTS, (Ch. 14) call prior to entering channel.

Outer anchorage off Hayward Point, open to NE.

Pilotage for >100 NRT (*Not required RCS in 12/14)

Long channel marked with beacons, tidal currents to 2.5 knots.  If possible plan ETA at entrance to catch last 2 hours of flood. Verify this with harbormaster.

Dunedin Tides

--Subtract ½ hour (H) and 1:20 (L) for entrance (Spit Wharf)

Dunedin Harbormaster is also contact for wharfage:

Hugh Marshall 				HMarshall@portotago.co.nz

Pilots Office at Port Otago Ltd             	+64 3 472 9883

pilots@portotago.co.nz

Prior to arrival the vessel may make contact with Harbour Control by VHF: Tel (03) 472 9882 harbourcontrol@portotago.co.nz

Note that time change occurs on 1 April (ZD -13 to -12)