C322D Passage Plans

1. Sunday, August 3: Woods Hole to XXX

Tides (EDT)

Woods Hole  H:0353 (+1.18)  L:0949 (+0.78) H:1644 (+1.87)

Boston  H: 0713  L: 1301  H: 1926

Tidal Currents (EDT)

Woods Hole

SLACK: 0642(H) MAX: 0834 (E) SLACK: 1231(L)

Vineyard Sound (1.5 nm E of Tarpaulin Cove)

SLACK: 0752(H) MAX: 1053 (E) SLACK: 1412(L)

Ferries:

0630: NAN Departs.

0645: MAR Arrives.

0700: MAR Departing.

0715: Aquinna Arrives

0730: Aquinna Depart (?)

0745: IHM Arrives

ETD: 0700+, just following MAR departure

Notes: MAR Departed 0720 on Friday, just before late  Aquinna arrival

Weather:

0600: All hands prepare for departure

- Single up

- Strike gangway

- Secure shore power

- Finalize pre-departure checklists, anchors ready, etc.

- Radios: 16, 13

T-30: All hands pre-departure brief and "T-Boat" emergency instructions

-15: Start main engine, conduct propulsion check

-00: Make departure after security call. Confirm ferry traffic.

Ferries:

T+15: Outbound, Woods Hole Passage, 182°M. Elliot has con, Karen is Navigator, Ryan and Lisa Anne run the deck. Staff at helm to start.

*Create a written rotation of staff to cover helm, lookout, and boat checks during transit.

0730-1030: Proceed to anchorage (wx permitting) per routes laid out on RP. 		          		          Currents expected en route:

Watch condition 3 (Master, conning officer, designated lookout)

Emergency responses per base station bill.

1030: Anchored, Menemsha, lunch followed by station bill and drill walk-throughs. 	maintain staff rotation for coverage of boat checks and anchor watches through day

1900: Set anchor watches after evening meal

2. AUGUST 4

Coastal Weather

Synopsis for Massachusetts and Rhode Island coastal waters...

Vineyard Sound-

TODAY...

TONIGHT...

MON...

MON NIGHT...

Buzzards Bay-

Cape Cod Bay-

TIDES AND CURRENTS AUGUST 4

Boston: 	H 0811   L 1401   H 2021

Cape Cod Canal Entrance:  Slack at 1248 (easterly flood starting)

Vineyard Sound: Slack at 0811, Ebb starting

Quick's Hole: Slack at 0811, 2.4 knot northerly set by 0911

PASSAGE PLAN FOR AUGUST 4

0700 Conduct station at anchor, Menemsha

0800 Under way for Buzzards Bay via Quick's Hole

0800-1200 Transit Buzzards Bay per track on RPCN, arrive at Canal entrance c. 1200

- Watch condition 3 (Watch officer plus captain or additional mate, posted lookout)

- Sail/Engine plan per conditions

1200-1400 Cape Cod Canal Transit

1400 Exit canal and proceed northeast across Cape Cod Bay, keeping east of coastwise traffic. Overnight passage plan TBD based on conditions. Possibilities include standing. offshore into Gulf of Maine or anchorage at Provincetown.

NOTES

CAPE COD CANAL (US COAST PILOT VOL. 2, CH. 5)

"Cape Cod Canal Control" Monitors VHF Ch. 16, 14, 13, 12.

Call-In required (CH. 16) for clearance no less than 15 minutes prior to transit. (Have Vessel Particulars ready.)

Staff should be at helm to steer, or at minimum coach any student helmspeople.

Anchors to be ready throughout this leg

9. August 11

1300: Arrive Dyer's Dock

HWS Woods Hole 1248

Ferries: 	1220 WH Departs

1245 IHM Arrives

1315 IHM Departs